Protective Case for use with the

Arrow® VPS G4™ Interface

Setup Instructions

The enclosed protective case is used to mount the 
Arrow® VPS G4™ Interface onto a GCX, or similar type, cart.  These instructions describe how to insert the Interface into the case and then mount the assembly onto a 75 mm x 75 mm VESA-compatible mounting bracket (not included).

Please refer to the Arrow VPS G4 Device Operator’s Manual for complete instructions on using the Arrow VPS G4 Device.

List of Items Provided:

Four (4) Screws – Screw, pan head Phillips, M4x16mm SST

Four (4) Nuts – Hex Nut, Locking, M4 x 0.7mm Thread

One (1) Tablet Case – Kensington Enclosure for Android Tablet

List of Items Needed But Not Provided:

Wrench – M4 or 9/32”

Phillips Head Screwdriver

Android Tablet – Provided with VPS G4 Device

VESA plate – Provided with GCX Cart

Attaching Case to the VESA Mounting Bracket

- Slide out tab on rear of the Case (Figure 1).

Figure 1: Slide the Tab Away From Case

- Pull apart the top and bottom sections of the Case to separate them (Figure 2).

Figure 2: Separate the Case Pieces

- Align the four holes on large section of Interface Case with the four inner holes on the 75 mm x 75 mm VESA mounting bracket, ensuring the USB cable port is on the right side.

- Insert the four screws (provided) through the mounting bracket and into the case (Figure 3).

Figure 3: Insert the Four Screws

- Using a wrench, attach and tighten the four nuts (provided) to the screws on the inside of the Case (Figure 4).

Figure 4: Attach Nuts to the Four Screws

Do NOT overtighten the nuts as this can result in a cracked case. They should be tightened just to the point of there being no gap between the nut and the case.

Assembling Interface and Case

- Slide the Interface into the Case (Figure 5).

Figure 5: Slide Interface into Case

- Replace the smaller section of the Case and slide the tab firmly into place – it should “click” when it is in place (Figures 6 and 7).

Figure 6: Replace the Smaller Section

Figure 7: Slide Tab Firmly Into Place

- The Interface is now secure and ready to be used.

NOTE: For additional security, a compatible locking device can be purchased separately, either from Teleflex or via an alternate online vendor:

ClickSafe® Keyed Laptop Lock, part # K64637WW.

Teleflex, the Teleflex logo, Arrow, and VPS G4 are trademarks or registered trademarks of Teleflex Incorporated or its affiliates, in the U.S. and/or other countries.

© 2016 Teleflex Incorporated. All rights reserved.

ClickSafe is a trademark or registered trademark of ACCO Brands Corporation and its Subsidiaries.

Distributed by:

Teleflex Medical

IDA Business and Technology Park

Dublin Road, Athlone

Co. Westmeath, Ireland

Teleflex US Customer Service (product and accessory orders):

Tel: 	+1.866.246.6990

Fax: 	+1.866.804.9881

E-Mail: 	cs@teleflex.com

Teleflex US  VPS Technical Support:

Tel: 	+1.877.236.6869

Email: 	vpstechsupport@teleflex.com

Web site:	www.arrowvascular.com/vps

www.teleflex.com